Network+ N10-009 Cram Sheet

Ports

| Protocol | Port |
| --- | --- |
| File Transfer Protocol (FTP) | 21 |
| Secure File Transfer Protocol (SFTP) | 22 |
| Secure Shell (SSH) | 22 |
| Telnet | 23 |
| Simple Mail Transfer Protocol (SMTP) | 25 |
| Domain Name System (DNS) | 53 |
| Dynamic Host Configuration Protocol (DHCP) | 67/68 |
| Trivial File Transfer Protocol (TFTP) | 69 |
| Hypertext Transfer Protocol (HTTP) | 80 |
| Network Time Protocol (NTP) | 123 |
| Simple Network Management Protocol (SNMP) | 161 |
| Lightweight Directory Access Protocol (LDAP) | 389 |
| Hypertext Transfer Protocol Secure (HTTPS) | 443 |
| Server Message Block (SMB) - File Share | 445 |
| Syslog | 514 |
| Simple Mail Transfer Protocol Secure (SMTPS) | 465 |
| Lightweight Directory Access Protocol over SSL (LDAPS) | 636 |
| Remote Desktop Protocol (RDP) | 3389 |

OSI Model

| # | Layer Name | Data Type |
| --- | --- | --- |
| 7 | Application | Data |
| 6 | Presentation | Data |
| 5 | Session | Data |
| 4 | Transport | Segments |
| 3 | Network | Packets |
| 2 | Data Link | Frames |
| 1 | Physical | Bits |

- Encapsulates as you move down  Mnemonic: All People Seem To Need Data Processing

- Decapsulates as you move up  Mnemonic: Please Do Not Throw Sausage Pizza Away

Traffic Types

| Unicast | One-to-one communication |
| --- | --- |
| Multicast | One-to-many communication |
| Anycast | One-to-nearest communication |
| Broadcast | One-to-all communication |

Network Topologies

| Mesh | connections between every device |
| --- | --- |
| Star | All connected to a central hub |
| Hybrid | Combines different topologies |
| Bus | All connected to a central cable |
| Ring | Circular data path with devices connected in a loop |
| Point-to-Point | connection between two devices |

Fiber Optic Connectors

| Subscriber Connector (SC) | Square connector with a push-pull locking mechanism. |
| --- | --- |
| Local Connector (LC) | Small connectors used for high-density connections, has a Lip |
| Straight Tip (ST) | Used in multi-mode fiber networks. Has a long Tip. |
| Multi-fiber push on (MPO) | Large connector for high volume transfer |

IPv4 Addresses

| Public IP addresses | Used on the internet, assigned by the ISP | - 8.8.8.8 - 1.1.1.1 - 172.217.0 - 13.35.0.1 |
| --- | --- | --- |
| Private IP addresses | Used within private networks | - 10.0.0.0 - 10.255.255.255 - 172.16.0.0 -172.31.255.255 - 192.168.0.0 -192.168.255.255 |

Network Routing Protocols

| Protocol | Domain | Mechanism | Convergence |
| --- | --- | --- | --- |
| RIP | Intra-AS | Distance Vector | Slow |
| EIGRP | Intra-AS | Distance Vector | Very Fast |
| OSPF | Intra-AS | Link State | Fast |
| BGP | Inter-AS | Path Vector | Slow |

SDN Terms

| Application aware | Network adapts to app requirements |
| --- | --- |
| Zero-touch provisioning | Automatically configures devices |
| Transport agnostic | Allows seamless transition between transport methods |

VLAN Configuration

| Native VLAN | Manages untagged traffic |
| --- | --- |
| Voice VLAN | Separates voice calls from other traffic |
| 802.1Q tagging | Identifies the VLAN the frame belongs to |
| Link Aggregation | Combining multiple Ethernet links into a single, logical link. |

Interface Errors

| Error Type | Description |
| --- | --- |
| CRC Errors | Code that detects frame corruption |
| Giant | Frame size 9,000 bytes |
| Runt | Frame is smaller than MTU |
| Encapsulation Error | Incorrect protocol settings |

Physical Installations

| Location | MDF is main (Ex: Broward Co.), IDF is intermediary (Nova HS). |
| --- | --- |
| Rack Size | Optimizes floor space and power efficiency |
| Port-side exhaust/intake | Hot aisle/cold aisle setup for airflow management |
| Cabling | FDP is for fiber cables, patch panels for diverse cables. |
| Power | PDU distributes power, UPS provides backup power |
| Environmental | Fire suppression, temperature, and humidity controls |

Service Models

| Software as a Service | SW and HW managed by provider (e.g. canvas) |
| --- | --- |
| Platform as a Service | Platform for developers to build upon |
| Infrastructure as a Service | You manage apps and DB, provider manages HW |
| On Premise | You manage all components |

Wireless Standards

| Wi-Fi Gen | IEEE | Frequency | Speed |
| --- | --- | --- | --- |
| Wi-Fi | 802.11 | 2.4 Ghz | 2 Mbps |
| Wi-Fi 1 | 802.11b | 2.4 | 11 Mbps |
| Wi-Fi 2 | 802.11a | 5 | 54 Mbps |
| Wi-Fi 3 | 802.11g | 2.4 | 54 Mbps |
| Wi-Fi 4 | 802.11n | 2.4 & 5 | 600 Mbps |
| Wi-Fi 5 | 802.11ac | 5 | 6.93 Gbps |
| Wi-Fi 6 | 802.11ax | 2.4, 5 & 6 | 14 Gbps |

Wired Standards

| Type | Name | Speed | Max Length |
| --- | --- | --- | --- |
| 10BASE-T | Ethernet | 10 Mbps | 100m |
| 100BASE-T (T = Copper) | Fast Ethernet | 100 Mbps | 100m |
| 1000BASE-LX (L = Light) | Gig Ethernet | 1000 Mbps | 5000m (fiber) |
| 1000BASE-T | Gig Ethernet | 1000 Mbps | 100m |
| 10GBASE-T | 10 Gig Ethernet | 10 Gbps | 100m |

Documentation

| Physical Diagram | Shows physical aspects (cables and ports) |
| --- | --- |
| Logical Diagram | Shows information flow |
| Rack Diagram | Provide documentation for installation/changes. |
| Cable Map | Contains physical cable/fiber layout |
| Heat Map | Identifies wireless signal strengths. |

- End-Of-Life stops support but continues to update/patch.

- End of Support stops updating/patching the software entirely.

Disaster Recovery Metrics

| Metric | Use |
| --- | --- |
| RPO | Maximum data loss a company can afford |
| RTO | Acceptable time to restore a network. |
| MTTR | Average time to repair a system. |
| MTBF | Average time between system failures |

Plans/Procedures

| Plan | Purpose |
| --- | --- |
| Change Management | Manages/tracks changes to network settings |
| Disaster Recovery | Procedures for restoring a network after a disaster |
| Business Continuity | Ensures business ops. continue after a disaster |
| Standard Operating Procedure (SOP) | Step-by-step instructions for routine operations |

DNS Record Types

| A | Shows the IPv4 address for a domain. |
| --- | --- |
| AAAA | Points to the IPv6 address for a domain. |
| CNAME | Points an alias domain to another domain (ex: microsoft.com for www.microsoft.com) |
| MX | Routes emails to the correct mail server. |
| TXT | Stores information about a domain in text format. |
| NS | Specifies the authoritative DNS server for a domain. |
| PTR | Provides a domain name for reverse lookup (IP Domain URL) |

Zone Types

| Forward/Reverse | Maps domain names to IPs and vice versa |
| --- | --- |
| Authoritative/Non-Authoritative | Official records vs. cached queries |
| Primary vs. Secondary | Primary is read-write, Secondary is read-only |
| Recursive | Servers query each other to resolve requests. |

Authentication Protocols

- RADIUS

  - Network device authentication VPN

  - Ports 1812/1813

- TACACS+

  - Cisco's authentication system

  - Port 49

- Kerberos

  - Windows authentication for SSO

  - Port 88

Dynamic Addressing (DHCP)

| Feature | Details |
| --- | --- |
| Reservations | Reserve IPs for specific devices |
| Options | Additional settings (e.g., subnet mask, DNS) |
| Relay/IP helper | Relay DHCP requests across subnets |
| Exclusions | IPs outside the DHCP scope |
| SLAAC | Devices configure themselves with IPv6 |

Network Access Methods

Site-to-Site VPN: Connects two sites securely over the Internet.

Client-to-Site VPN: Connects users to a remote network securely.

- Clientless: Access via web browser without VPN client.

Network Attacks

| VLAN Hopping | Exploiting VLAN tag vulnerabilities |
| --- | --- |
| MAC Flooding | Overwhelming switch’s MAC table to cause broadcast mode |
| ARP Poisoning | Forging ARP requests to redirect to an attacker’s device. Often the result of ARP Spoofing. |
| ARP Spoofing | Mimicking a MAC address to appear legitimate. |
| DNS Poisoning | Modifies DNS cache to redirect traffic to malicious sites. |
| DNS Spoofing | Sending forged DNS responses to a client. |
| Evil Twin | Malicious WAP that mimics a legitimate one to steal data |
| On-Path attack | Intercepting traffic between 2 nodes |

Troubleshooting Methodology

1. Identify the Problem

2. Establish a Theory of Probable Cause

3. Test The Theory

4. Establish a plan of action

5. Implement solution or escalate.

6. Verify full system functionality

7. Documenting findings

Command Line

| ping | Tests reachability of another host with ICMP requests |
| --- | --- |
| tracert | Shows route and time to reach a destination (traceroute for Linux) |
| nslookup | Resolves domain names to IP addresses (dig in Linux/Unix) |
| tcpdump | Captures and inspects network traffic |
| netstat | Lists active connections and network statistics |
| ipconfig | Displays/configures IP settings (ifconfig in Linux) |
| arp | Interacts with ARP cache |
| nmap | Scans hosts and open ports |

Networking Device Commands

| show mac-address-table | Displays learned MAC addresses |
| --- | --- |
| show route | Shows the current routing table |
| show interface | Displays interface status/statistics |
| show config | Displays current device configuration |
| show arp | Shows ARP cache (IP-to-MAC address mappings) |
| show vlan | Lists configured VLANS and their status |
| show power | Displays power supply and usage information |

Time Protocols

- NTP: Synchronizes clocks across devices in a network.

- PTP (Precision Time Protocol): More accurate than PTP.

- NTS (Network Time Security): Secure time synchronization using cryptography

Connectivity Tools

| Tool | Use |
| --- | --- |
| Cable Crimper | Attaches connectors to cables |
| Punchdown Tool | Terminates wires into punchdown blocks |
| Tone generator | Detects cable path with a traceable tone (“Finds” cables) |
| OTDR | Tests fiber optic cable integrity |
| Cable Tester | Tests cables for proper wiring and transmission |
| Spectrum Analyzer | Analyzes signal frequencies to identify interference |

Common Network Issues

| Issue | Problem | Solution |
| --- | --- | --- |
| Collisions/ Broadcast Storm | Network congestion | Configure VLANS, enable STP |
| Duplicate IPs | 2 devices have the same IP address | Configure DHCP or assign static IPs |
| DHCP Scope Exhaustion | No available IPs for other devices | Change the DHCP scope or reserve IPs |

Wireless Security

| Security Type | Features | Security Level |
| --- | --- | --- |
| WPA3 | Latest Standard Strongest Encryption | Highest |
| WPA2 | Still widely used and supported AES encryption | High |
| WPA | Older standard with TKIP encryption | Medium |
| WEP | Easily broken basic encryption, Don’t use | Low |

Basic CIDR Subnetting:

- /24 = 256 hosts

- /25 = 128 hosts

- /26 = 64 hosts

- /27 = 32 hosts

- /28 = 16 hosts